Design Strategy for Project 2

Haibei Zhang  Section 2

Block Diagram:

Office Control Box:

- AVAILABLE (output)
The AVAILABLE signal is based on the 3 (or 2) ENABLE signals on the data transfer path. The 3 ENABLE will be selected from all 5 ENABLE signals from the bus, among which 2 from ENABLE a, b, d, e and 1 is ENABLE c. 

- SRC1, SRC0 will select one ENABLE from a, b, d, e  (a 4 input 1 bit mux will be used)
- DST1, DST0 will select one ENABLE from a, b, d, e  (a 4 input 1 bit mux will be used)
- SRC1⊕DST1 will determine whether ENABLE c will be considered (use a 3-state buffer)

The product of above 3 inputs will be the AVAILABLE

- CONNECT (input)
Once CONNECT is asserted, the DST and SRC will determine which ENABLE and DIR signals will be set as active low. (use a 74_541 to connect/disconnect DST and SRC)

Setting of 5 ENABLE lines:
- SRC1, SRC0 will set one ENABLE line from a, b, d, e (use 1/2 74_139 2-to-4 decoder)
- DST1, DST0 will set one ENABLE line from a, b, d, e (use 1/2 74_139 2-to-4 decoder)
(totally, a 74_139 and a 74_541 will be used so that the Enable lines are either set active low or left Hi-Z)
- SRC1⊕DST1 will determine whether ENABLE c will be set (use a 3-state buffer)

Setting of 5 DIR lines:
- SRC1, SRC0 will set one DIR line from a, b, d, e (use 1/2 74_139 2-to-4 decoder)
- DST1, DST0 will set one DIR line from a, b, d, e (use 1/2 74_139 2-to-4 decoder)
(totally, a 74_139 and a 74_541 will be used so that the DIR lines are either set active low or left Hi-Z)
- If SRC1⊕DST1 is 1 and DST1 is 1, set the DIR c low (use a 3-state buffer).

Switch Box:
Five 74_245 transceivers will be used for a, b, c, d, e