An 8-Bit CPU Control Circuit Design

CSE 207 Final Project

Section 2

Haibei Zhang

Dec 10, 2002

http://www.haibei.net/cse207/finalproject.pdf

haibei@haibei.net

I. Objectives

The objective of this project is to design a control circuit for the Robertics WFBCSC II CPU, a 8-bit Von Neumann machine.

The inputs and outputs are:

| Input | Output (Abbreviation) |
| --- | --- |
| Reset (Active High) | PC_TO_BUS (Active Low) (PB) |
| ZCC (Zero Condition Code) | BUS_TO_PC (Active Low) (BP) |
| Clock | BUS_TO_MAR (Active Low) (BMAR) |
| I7 (Instruction 8th bit) | BUS_TO_MDR (Active Low) (BMDR) |
| I6 (Instruction 7th bit) | MDR_TO_BUS (Active Low) (MDRB) |
| I5 (Instruction 6th bit) | R (Active Low) |
| I4 (Instruction 5th bit) | W (Active Low) |
| I3 (Instruction 4th bit) | BUS_TO_AIR (Active Low) (BAIR) |
| I2 (Instruction 3rd bit) | AOR_TO_BUS (Active Low) (AORB) |
| I1 (Instruction 2nd bit) | BUS_TO_IR (Active Low) (BI) |
| I0 (Instruction 1st bit) | R3 (Register Address 4th bit) |
|  | R2 (Register Address 3rd bit) |
|  | R1 (Register Address 2nd bit) |
|  | R0 (Register Address 1st bit) |
|  | REG_TO_BUS (Active Low) (RB) |
|  | BUS_TO_REG (Active Low) (BR) |
|  | S3 |
|  | S2 |
|  | S1 |
|  | S0 |
|  | CIN (Active Low) (CN) |

II. Design Procedures

1. Design the state diagram for the 8-bit instruction fetching. (Each state is assigned a 3-bit binary number)

000*

001

010

011

Reset’

Reset’

Reset’

Decode and execute the instruction

* If Reset is asserted, any state goes to the 000 state. These transitions are not shown here.

Set PC->MAR

Set PC+1->AOR

Set AOR->PC

Set MEM->MDR

Set MDR->IR

Clear PC

100

Reset’

IR Loaded

Use IR as input

Reset’ •I7’ (If I7=0, that’s a 8-bit instruction!)

Reset’

2. Design the complete state diagram for both 8-bit and 16-bit instruction fetching. (Each state is assigned a 3-bit binary number)

000*

001

010

011

Reset’

Reset’

Reset’

Decode and execute

* If Reset is asserted, any state goes to the 000 state. These transitions are not shown here.

** If decoder finds an undefined instruction from IR, then it goes to the error state (initial state).

Set PC->MAR

Set PC+1->AOR

Set AOR->PC

Set MEM->MDR

Set MDR->IR

Clear PC

100

Reset’

Idle state

(IR loaded)

Reset’ •I7’

101

110

Set PC->MAR

Set PC+1->AOR

Set AOR->PC

Set MEM->MDR

Reset’ •I7

Reset’

Decode and execute

111

Execution

successful

Idle state

Execution

Failed**

(extra credit)

Reset’

3. Design the decoder

At state 100 and 111 shown above, IR is ready to input instruction into the circuit. I7 is used to determine whether the instruction is 8-bit or 16-bit (at state 100). I6, I5, I4 are used to choose a circuit which handles the execution of the instruction. (The 3 inputs of the decoders.) Therefore two 74138 decoders are used for 8-bit and 16-bit instructions. For 8 bit instructions, the decoder is enabled when state is 100, Reset=0, I7=0; for 16 bit instructions, the decoder is enabled when state is 111 and Reset=0. Once the decoder is enabled, it decodes I6, I5, I4, then sets one of its output pin active (the Start signal for the execution circuit), which triggers that execution to run.

4. Design the execution circuit (instruction processor)

Each instruction has its own circuit, which are also state machines. These machines are normally staying in a standby state. Once it receives a “Start” signal from the decoder, it starts running.

When the instruction-fetching machine reaches state 100 and 111, the next state will take instruction processor’s output into consideration. When the instruction is still in processing, it stays at 100 or 111; when the instruction is finished, it goes to 001; when the instruction encounters error, it goes to 000.

Therefore, the execution circuit should have these 2 outputs: Finished_L, Error_L. If both of them are inactive, the machine is processing an instruction.

Following are transition diagrams for each instruction:

ADDR (Opcode: 0010)

00*

01

10

11

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

*** “RI3I2I1I0’ ” means “data at register address I3 I2 I1 I0’ ” where I0’ is I0 inverted

Set RI3I2I1I0’->AIR***

Set AIR+ RI3I2I1I0->AOR**

Set AOR-> RI3I2I1I0’

Set Finished8_L Active

Standby

1

SUBR (Opcode: 0011)

00*

01

10

11

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

*** “RI3I2I1I0’ ” means “data at register address I3 I2 I1 I0’ ” where I0’ is I0 inverted

Set RI3I2I1I0’->AIR***

Set RI3I2I1I0’- AIR ->AOR***

Set AOR-> RI3I2I1I0’

Set Finished8_L Active

Standby

1

INCR (Opcode: 0100)

00*

01

11

Reset’•Start

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set RI3I2I1I0 + 1->AOR**

Set AOR -> RI3I2I1I0

Set Finished8_L Active

Standby

1

DECR (Opcode: 0101)

00*

01

11

Reset’•Start

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set RI3I2I1I0 - 1->AOR**

Set AOR -> RI3I2I1I0

Set Finished8_L Active

Standby

1

BR (Opcode: 0110)

0*

1

Reset’•Start

* If Reset is asserted, any state goes to the 0 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set RI3I2I1I0 ->PC**

Set Finished8_L Active

Standby

1

BRZ (Opcode: 0111)

00*

01

Reset’•Start•ZCC

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set RI3I2I1I0 ->PC**

Set Finished8_L Active

Standby

1

11

Set Finished8_L Active

Reset’•Start•ZCC’

LOAD (Opcode: 1000)

00*

01

10

11

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set MDR->MAR

Set MEM->MDR

Set MDR-> RI3I2I1I0 **

Set Finished16_L Active

Standby

1

LDI (Opcode: 1001)

0*

1

Reset’•Start

* If Reset is asserted, any state goes to the 0 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set MDR -> RI3I2I1I0**

Set Finished16_L Active

Standby

1

ADD (Opcode: 1010)

000*

001

010

011

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 000 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set MDR->MAR

Set MEM->MDR

Set RI3I2I1I0 -> AIR **

Set AIR + MDR ->AOR

Standby

1

100

Set AOR -> RI3I2I1I0

Set Finished16_L Active

SUB (Opcode: 1011)

000*

001

010

011

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 000 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set MDR->MAR

Set MEM->MDR

Set MDR->AIR

Standby

1

100

Set AOR -> RI3I2I1I0

Set Finished16_L Active

101

Set RI3I2I1I0 -AIR ->AOR

**

STORE (Opcode: 1100)

00*

01

10

11

Reset’•Start

Reset’

Reset’

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

** “RI3I2I1I0” means “data at register address I3 I2 I1 I0”

Set MDR->MAR

Set RI3I2I1I0 ->MDR**

Set MDR-> MEM

Set Finished16_L Active

Standby

1

B (Opcode: 1110)

0*

1

Reset’•Start

* If Reset is asserted, any state goes to the 0 state. These transitions are not shown here.

Set MDR ->PC

Set Finished16_L Active

Standby

1

BZ (Opcode: 1111)

00*

01

Reset’•Start•ZCC

* If Reset is asserted, any state goes to the 00 state. These transitions are not shown here.

Set MDR  ->PC

Set Finished16_L Active

Standby

1

11

Set Finished16_L Active

Reset’•Start•ZCC’

UNDEFINED INSTRUCTIONS (Opcode: 0000, 0001, 1101)

0*

1

Reset’•Start

* If Reset is asserted, any state goes to the 0 state. These transitions are not shown here.

Set Error_L Active

Standby

1

(For extra credit!)

III. Transition Lists, Excitation Equations (D Flip-Flop), Output Equations (This part was finished before Robert introduced MSI Counter. In practice, I will use MSI counter rather than D Flip-Flop.)

The Main Circuit (Instruction Fetching Cycle)

a) State – Next State transition list:

| Q2Q1Q0 | Reset (Active High) |  |
| --- | --- | --- |
|  | 1 | 0 |
| 000 | 000 | 001 |
| 001 | 000 | 010 |
| 010 | 000 | 011 |
| 011 | 000 | 100 |
| 100 | 000 | 001 if I7=0, Finished8_L=0 (go to next cycle) |
|  |  | 000 if I7=0, Error_L=0 (go to error state) |
|  |  | 100 if I7=0, Error_L=1, Finished8_L=1 (waiting) |
|  |  | 101 if I7=1 (go on fetching a 16-bit inst.) |
| 101 | 000 | 110 |
| 110 | 000 | 111 |
| 111 | 000 | 001 if Finished16_L=0 (go to next cycle) |
|  |  | 000 if Error_L=0 (go to error state) |
|  |  | 111 if Finished16_L=1, Error_L=1 (waiting) |
|  | D2D1D0 (Q2*Q1*Q0*) |  |

b) Excitation Equation:

D2=Reset’•(Q2’•Q1•Q0 + Q2•Q1’•Q0’•(I7 + I7’•Finished8_L•Error_L) + Q2•Q1’•Q0 + Q2•Q1•Q0’ + Q2•Q1•Q0•Finished16_L•Error_L)

D1=Reset’•(Q1•Q0’ + Q1’•Q0 + Q2•Q1•Q0•Finished16_L•Error_L)

D0=Reset’•(Q2’•Q0’ + Q1•Q0’ + Q2•Q1’•Q0’•(I7’•Finished8_L’ + I7) + Q2•Q1•Q0•(Finished16_L’ + Finished16_L•Error_L))

c) State – Output table

Part 1:

| Q2Q1Q0 | PB | BP | BMAR | BMDR | MDRB | R | W | BAIR | AORB | BI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 001 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 010 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 011 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 100* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 101 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 110 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 111* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Part 2:

| Q2Q1Q0 | R3 | R2 | R1 | R0 | RB | BR | S3 | S2 | S1 | S0 | CN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 001 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 010 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 011 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 100* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 101 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 110 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 111* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

*In the state 100 and 111, outputs are controlled by execution circuits (the main circuit is disconnected from outputs). Therefore, use Hi-Z signal instead of 1 in practice.

d) Output Equations:

PB=BMAR=S3=S2=S1=S0=CN=Q1 + Q0’

BP=R=AORB=Q1’ + Q0

MDRB=BI=Q2 + Q1’ + Q0’

All other outputs are not affected by this circuit.

IV. Transition Lists and Equations (Using MSI Counter 74x163)

The Main Circuit (Instruction Fetching Cycle)

| Step | Reset (Active High) |  |
| --- | --- | --- |
|  | 1 | 0 |
| St0 | St0 | St1 |
| St1 | St0 | St2 |
| St2 | St0 | St3 |
| St3 | St0 | St4 |
| St4 | St0 | St1 if Finished8_L=0 (go to next cycle) |
|  |  | St0 if Error_L=0 (go to error state) |
|  |  | St4 if I7=0, Error_L=1, Finished8_L=1 (waiting) |
|  |  | St5 if I7=1 (go on fetching a 16-bit inst.) |
| St5 | St0 | St6 |
| St6 | St0 | St7 |
| St7 | St0 | St1 if Finished16_L=0 (go to next cycle) |
|  |  | St0 if Error_L=0 (go to error state) |
|  |  | St7 if Finished16_L=1, Error_L=1 (waiting) |
|  | Next Step |  |

The CLR input of counter is set:

CLR=Reset’

The LOAD of counter is set:

LOAD=Step4•Step7

Input values C, B, A of the counter need to be set at Step4 and Step7 (by converting from D Flip-Flop’s excitation equations):

C=Error_L• (I7+Finished8_L) • (I7’+Finished16_L)

B=Finished16_L•Error•Step7’

A=Error_L• (Step4+I7+Error_L’+Finished8_L’)

State(Step) – Output table

Part 1:

| Step | PB | BP | BMAR | BMDR | MDRB | R | W | BAIR | AORB | BI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| St0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St2 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| St3 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| St4* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St5 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St6 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| St7* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Part 2:

| Step | R3 | R2 | R1 | R0 | RB | BR | S3 | S2 | S1 | S0 | CN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| St0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| St2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St4* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St5 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| St6 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| St7* | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Instruction execution circuits

A similar MSI counter and decoder is used for each instruction processor. I don’t list all state-output tables here because it’s easy to look at the state diagram and decide which outputs need to me set.

The three undefined instructions are also handled by resetting the state of the instruction-fetching circuit. (For extra credit.)

V. Testing

Testing of this CPU involves a circuit level testing and a system level testing.

- Circuit level testing (See file ProcessorControlCircuit.cct.)

The control circuit alone can be tested by manually setting inputs and observe the values of each output.

First, find the “testing area” and attach all binary switches to the input. (I have to detach them so that the circuit can work in the whole system.) Now, reset the circuit by setting Reset=1. Then input different I7I6I5I4 and see the state of the circuit and all output values at each state. The testing is easy because every state and output should be same as described in the diagrams in this document.

I have tested all 16 types of instructions (including undefined). All observed values are expected. The circuit is working fine at this level.

- System level testing (See file processor.cct)

Now this circuit is transferred into a symbol and installed in the main CPU architecture. I have finished the testing of following list of instructions:

| Memory Addr. | Instruction /Hex value | Comments |
| --- | --- | --- |
| 00000000 | 1001 1001 /99 | LDI (load a value to a register) |
| 00000001 | 0010 1100 /2C | The value of last LDI inst. (2c) |
| 00000002 | 0100 1001 /49 | INCR (increase that value) |
| 00000003 | 1001 1000 /98 | LDI (load another value) |
| 00000004 | 1000 0000 /80 | The value of last LDI inst. (80) |
| 00000005 | 0011 1000 /38 | SUBR between 2 register values |

The final result (Register 1000’s value) is expected to be 53 (Hexadecimal).

Procedures:

- Set control circuit’s Reset=1 (Resent is active high)

- Enable the 2 bus drivers at the bottom (the 8-bit 74x541 to set data, another 4-bit one to set control signals) so that you can manually set values.

- Set pc to 00000000

- Load memory with above instructions one by one, from memory address 00000000 first. This is a very tedious job. For each instruction, I need to (1) load memory address to MAR (2) load instruction into MDR (3) assert W.

- Once all instructions have been input into the memory, disable the 2 bus drivers (disconnect the manual input), and set Reset=0. Show time begins:

Here are the values read from Hex probes, each row indicates a clock cycle.

| BUS | PC | MAR | MDR | AIR | AOR | IR |
| --- | --- | --- | --- | --- | --- | --- |
| xx | 00 | Last | Last | xx | xx | Last |
| 00 | 00 | Last | Last | Xx | Xx | Last |
| 01 | 00 | 00 | Last | xx | 01 | Last |
| 99 | 01 | 00 | 99 | xx | xx | Last |
| xx | 01 | 00 | 99 | xx | xx | 99 |
| 01 | 01 | 00 | 99 | xx | xx | 99 |
| 02 | 01 | 01 | 99 | Xx | 02 | 99 |
| Xx | 02 | 01 | 2c | xx | xx | 99 |
| 2c | 02 | 01 | 2c | xx | xx | 99 |
| 02 | 02 | 01 | 2c | xx | xx | 99 |
| 03 | 02 | 02 | 2c | Xx | 03 | 99 |
| 49 | 03 | 02 | 49 | xx | xx | 99 |
| xx | 03 | 02 | 49 | Xx | Xx | 49 |
| 2c | 03 | 02 | 49 | xx | xx | 49 |
| 2d | 03 | 02 | 49 | Xx | 2d | 49 |
| 03 | 03 | 02 | 49 | Xx | Xx | 49 |
| 04 | 03 | 03 | 49 | Xx | 04 | 49 |
| 98 | 04 | 03 | 98 | xx | xx | 49 |
| xx | 04 | 03 | 98 | Xx | Xx | 98 |
| 04 | 04 | 03 | 98 | xx | xx | 98 |
| 05 | 04 | 04 | 98 | xx | 05 | 98 |
| xx | 05 | 04 | 80 | xx | xx | 98 |
| 80 | 05 | 04 | 80 | xx | xx | 98 |
| 05 | 05 | 04 | 80 | xx | xx | 98 |
| 06 | 05 | 05 | 80 | Xx | 06 | 98 |
| 38 | 06 | 05 | 38 | xx | xx | 98 |
| xx | 06 | 05 | 38 | Xx | Xx | 38 |
| 2d | 06 | 05 | 38 | xx | xx | 38 |
| 80 | 06 | 05 | 38 | 2d | Xx | 38 |
| 53 | 06 | 05 | 38 | 2d | 53 | 38 |

By now, the expected result (53) has appeared in the AOR and Bus!

VI. Discussions and Conclusions

The circuit level testing can only tell me whether the circuit is a good implementation of my logic. The system level testing is more important as it shows the machine’s ability to solve real problems. Due to the difficulty of I/O and other things that have to be done manually, it is hard to conduct a comprehensive testing. However, by running this randomly chosen set of instructions, the CPU has shown correctness in instruction fetching and execution. Miracle happens.