Relocatable Assembler, Linker, and Loader

With Java

CSE 258 Project 1

Haibei Zhang

Feb 6, 2003

- To Use This Program:

- Run the executable “proj1Win32.exe” or compile the source code file “main.java”. Make sure “input1.txt”, “input2.txt”, “input3.txt” are in the same folder.

- Choose a file name and click “load” to load a source 6502 file.

- Click “step 1” to convert it into relocatable code

- Input a 4-digit hexadecimal starting address and click “step 2” to convert it into relocated machine code

- Design Strategy

The purpose of this project is to convert 6502 assembly code into relocatable code (step 1) and relocated machine code (step 2).

In step 1, I first scan the source program, looking for symbols defined in the beginning of each instruction. Meanwhile, I scan each instruction and compute its length in bytes, then from the top to the bottom I assign each instruction a memory address starting from 0000. If an instruction has a symbol defined, an instance of Symbol class will be created (with its name and address) and added into the symbol table (implemented as a vector). For every instruction, an instance of Instruction class will be created and added into an Instruction Vector (each instruction has 2 properties: address and content).

Some instructions may refer to symbols. In the second pass, I scan all the instructions in the vector, and replace these symbol names with their corresponding address (this mapping has been stored in symbol table).

Finally, I print the instruction vector in the TextArea in the middle.

In step 2, I first add the starting address (obtained from an input TextField) to each instruction’s relocatable address. In the body of some instructions, there are also relocatable addresses which need to be updated. I scan every instruction and update all absolute addresses, immediate addresses are remained intact.

3. Results (In step 2, starting address is set 1F00)

a. input1.txt

| Source code | Relocatable (step 1) | Relocated (step 2) |
| --- | --- | --- |
| start LDY #$FF loop1 LDX #$FF loop2 DEX BNE loop2 DEY BNE loop1 RTS | 0000 LDY #$FF 0002 LDX #$FF 0004 DEX 0005 BNE $0004 0007 DEY 0008 BNE $0002 000A RTS | 1F00 LDY #$FF 1F02 LDX #$FF 1F04 DEX 1F05 BNE $1F04 1F07 DEY 1F08 BNE $1F02 1F0A RTS |

b. input2.txt

| Source code | Relocatable (step 1) | Relocated (step 2) |
| --- | --- | --- |
| mlt16 LDA #$00 STA $0026 STA $0027 LDX #$16 nxtbt LSR $0021 ROR $0020 BCC align LDA $0026 CLC ADC $0022 STA $0026 LDA $0027 ADC $0023 align ROR A STA $0027 ROR $0026 ROR $0025 ROR $0024 DEX BNE nxtbt RTS | 0000 LDA #$00 0002 STA $0026 0005 STA $0027 0008 LDX #$16 000A LSR $0021 000D ROR $0020 0010 BCC $0022 0012 LDA $0026 0015 CLC 0016 ADC $0022 0019 STA $0026 001C LDA $0027 001F ADC $0023 0022 ROR A 0023 STA $0027 0026 ROR $0026 0029 ROR $0025 002C ROR $0024 002F DEX 0030 BNE $000A 0032 RTS | 1F00 LDA #$00 1F02 STA $1F26 1F05 STA $1F27 1F08 LDX #$16 1F0A LSR $1F21 1F0D ROR $1F20 1F10 BCC $1F22 1F12 LDA $1F26 1F15 CLC 1F16 ADC $1F22 1F19 STA $1F26 1F1C LDA $1F27 1F1F ADC $1F23 1F22 ROR A 1F23 STA $1F27 1F26 ROR $1F26 1F29 ROR $1F25 1F2C ROR $1F24 1F2F DEX 1F30 BNE $1F0A 1F32 RTS |

c. input3.txt

| Source code | Relocatable (step 1) | Relocated (step 2) |
| --- | --- | --- |
| sqrt16 LDY #$01 STY $0022 DEY STY $0023 again SEC LDA $0020 TAX SBC $0022 STA $0020 LDA $0021 SBC $0023 STA $0021 BCC nomore INY LDA $0022 ADC #$01 STA $0022 BCC again INC $0023 JMP again nomore STY $0020 STX $0021 RTS | 0000 LDY #$01 0002 STY $0022 0005 DEY 0006 STY $0023 0009 SEC 000A LDA $0020 000D TAX 000E SBC $0022 0011 STA $0020 0014 LDA $0021 0017 SBC $0023 001A STA $0021 001D BCC $0030 001F INY 0020 LDA $0022 0023 ADC #$01 0025 STA $0022 0028 BCC $0009 002A INC $0023 002D JMP $0009 0030 STY $0020 0033 STX $0021 0036 RTS | 1F00 LDY #$01 1F02 STY $1F22 1F05 DEY 1F06 STY $1F23 1F09 SEC 1F0A LDA $1F20 1F0D TAX 1F0E SBC $1F22 1F11 STA $1F20 1F14 LDA $1F21 1F17 SBC $1F23 1F1A STA $1F21 1F1D BCC $1F30 1F1F INY 1F20 LDA $1F22 1F23 ADC #$01 1F25 STA $1F22 1F28 BCC $1F09 1F2A INC $1F23 1F2D JMP $1F09 1F30 STY $1F20 1F33 STX $1F21 1F36 RTS |

Based on observing of the results above, this program has shown its correctness in constructing symbol table, converting symbol name, and converting memory addresses.

4. Discussions

This program currently does not accept zero-page addressing. Zero-page addressing makes address-conversion complex. In step 2, an instruction like LDA $01 may become LDA $1F01, whose length changes from 2 byte to 3 byte. This change in its own length will affect all instruction’s address below. However, this problem can be solved by detecting the instruction’s change in length. If the address is no greater than FF before relocation, and greater than FF after relocation, its length is changed. All instructions after this one will have to increment their address by 1.